# Product Overview: SENSORSHLD1-EVK-101

# ROHM Multi-Sensor Shield

Above: Top view of ROHM SENSORSHLD0-EVK-101

25 September 2015, Revision A

# Introduction

The following document was written to provide a brief connection guide and general information about ROHM’s SENSORSHLD1-EVK-101.  Supplementary information including application examples and HW design files for this board can be found at the following repository link.

ROHM’s Multi-Sensor Shield GitHub Repository Page:  https://github.com/ROHMUSDC/ROHM_SensorPlatform_Multi-Sensor-Shield

ROHM’s SENSORSHLD1-EVK-101 is a shield evaluation platform that connects all ROHM sensor products onto a single board.  This shield uses standard Arduino shield interface pins; therefore can connect to any evaluation kit that has a shield interface header.  The shield contains the following sensors:

- Core Sensors:

  - ROHM BDE0600G – Analog Temperature Sensor

  - ROHM BM1383AGLV – Digital Barometric Pressure Sensor

  - ROHM BU52014HFV – Hall Switch Sensor (Omnipolar with Polarity Discrimination)

  - ROHM BM1422GMV – Magnetometer Sensor

  - KIONIX KX122 – Digital Accelerometer

  - KIONIX KMX62 – Digital Magnetometer and Accelerometer

  - KIONIX KXG03 – Digital Gyroscope and Accelerometer

  - LAPIS ML8511A – Analog UV Sensor

  - ROHM RPR-0521 – Digital Ambient Light Sensor and Proximity Sensor

  - ROHM BH1745 – Digital Color Sensor

- Special Functions:

  - KNOWLES SPM0423HD4H-WB – Digital Microphone

    - Primarily for use with NXP MCU Lineup

  - KIONIX KX122-1037, KX122-1048 – Accelerometer

    - For four corner Accelerometer algorithm development

    - Difference between 1037 and 1048 is the I2C register address scheme to control all 4 accelerometers using a single I2C master

# Key Differences between SHLD0 and SHLD1

- SENSORSHLD1-EVK-101 PN is the recommended evaluation board part number for new designs

- Removed Erroneous Jumpers

- Removed J5 to J11 and adjusted routing for J1 to J4

- Added ROHM BM1422GMV Magnetometer, 1.8V level shifter, and 1.8V LDO (for Magnetometer usage)

- Changed pressure sensor from BM1383GLV to BM1383AGLV.  (No HW change, but new PN has new FW I2C register mapping)

- PN Change from ML8511 to ML8511A (only chip labelling change)

- KIONIX Accelerometers

  - Removed original KX022

  - Changed corner Accelerometers to KX022 variation (smaller)

# General Board Specifications

For the specifications of the individual sensors on this shield board, please refer to the associated datasheet/application note that can be found on www.rohm.com

In general, this shield board will be powered by the V3.3 pin (Pin 4 of Header H4) on the shield board.  Please only connect a maximum of 3.3V to this pin as it is tied to the VDD of the full system and some of the sensors onboard cannot tolerate voltages greater than 3.3V.

# Hardware Explanation Section

- POWER ON LED: When the shield have 3.3V connected to the VCC Net, this LED will turn on

- BDE0600G Temperature Sensor

- BM1383GLV Pressure Sensor

- BU52014HFV Hall Effect Switch Sensor

- BM1422GMV Magnetometer Sensor

- KMX62 Accelerometer and Magnetometer Combo Sensor

- KXG03 Gyroscope and Accelerometer Combo Sensor

- ML8511/A UV Sensor

- RPR-0521 Combo Ambient Light Sensor and Proximity Sensor

- BH1745 RGB Color Sensor

- H4 – Standard Power Header

  - On the shield board, this header is only connected to pins 4, 7 and 8.  See section below for an explanation of the shield connector’s pin out

- H5 – Standard Analog In Header

  - See section below for an explanation of the shield connector’s pinout

- H7 – Standard Digital Header 1

  - See section below for an explanation of the shield connector’s pinout

- H10 - Auxiliary header for Microphone interface

- H6 – Standard Arduino Digital Header 2

  - See section below for an explanation of the shield connector’s pinout

- 0 Ohm Jumper Line for pins connected to H5

  - The purpose of these resistor shorts are to depopulate pin connections on the shield if the user decides to use a pass through path or needs to adjust pin functionality depending on the needed pin configuration

- 0 Ohm Jumper Line for pins connected to H6 and H7

  - The purpose of these resistor shorts are to depopulate pin connections on the shield if the user decides to use a pass through path or needs to adjust pin functionality depending on the needed pin configuration

- H3 – Expansion Header for ROHM Breakout Boards/Generic Interface (3 Pin – ADC/GPIO x1)

  - This header is an expansion header to connect any additional ADC/GPIO x1 based sensors that you may want to use

- H2 – Expansion Header for ROHM Breakout Boards/Generic Interface (4 Pin – ADC/GPIO x2)

  - This header is an expansion header to connect any additional ADC/GPIO x2 based sensors that you may want to use

- H1 - Expansion Header for ROHM Breakout Boards/Generic Interface (4 Pin – ADC/GPIO x2)

  - This header is an expansion header to connect any additional I2C + GPIOx1 based sensors that you may want to use

- KNOWLES SPM0423HD4H-WB Digital Microphone

- U13 Top Left Corner KX022 Accelerometer

- U16 Top Right Corner KX022 Accelerometer

- U14 Bot Left Corner KX022 Accelerometer

- U15 Bot Right Corner KX022 Accelerometer

# General Board Software Explanation

Software explanations will differ, depending on the application processor you plan to use.  Therefore, please see this shield’s repository for platform guides for using this shield:

https://github.com/ROHMUSDC/ROHM_SensorPlatform_Multi-Sensor-Shield

…/ROHM_SensorPlatform_Multi-Sensor-Shield/Platform Code/